Tables for Stem Cell Reviews and Reports submission

Manuscript: The Invisible Variables: Why Clonal Systems Are Not Immune to Environmental Confounding

Table 1. Environmental parameters in commercial GMP versus typical academic PSC research laboratories.

| Parameter | Commercial GMP status | Academic research lab status |
| --- | --- | --- |
| Temperature | 37C +/- 0.1C; validated and alarmed | 37C +/- 0.5C; rarely logged continuously |
| CO2 | 5% +/- 0.1%; calibrated regularly | 5% setpoint; calibration ad hoc |
| Humidity | Actively controlled and monitored | Seasonally coupled to outdoor air; rarely logged |
| VOCs | Carbon filtration; real-time monitoring in some facilities | Typically unmonitored |
| Ambient light | Controlled during processing | Variable by hood/incubator design |
| ELF-EMF | Shielded or characterized during qualification | Uncharacterized |
| Barometric pressure | Not typically controlled; logged in validated clean rooms | Not logged |
| Vibration | Isolated or characterized | Uncharacterized |

Table 2. Proposed IoT-enabled statistical roadmap for environmental profiling of PSC facilities.

| Phase | Objective | Statistical design | Deliverable |
| --- | --- | --- | --- |
| I. Passive monitoring | Establish baseline multi-parameter environmental variance | Longitudinal mixed-effects models; seasonality decomposition; control-chart anomaly detection | Facility-specific variance budget per environmental parameter |
| II. Retrospective crosswalk | Identify candidate variables that correlate with historical batch outcomes | Time-lagged regression with cross-validation; false-discovery rate control; sensitivity analyses for reagent lot and operator | ranked list of candidate variables |
| III. Controlled perturbation | Causally test high-priority variables | Blocked factorial or crossover designs; mixed models with random laboratory effect; equivalence testing for negative findings | Effect-size estimates and 95% confidence intervals |
| IV. Standardization | Deploy validated monitoring protocol across sites | Multi-center measurement-system analysis; Bland-Altman agreement; generalized linear mixed models for outcomes | Standard operating procedure and quality-control thresholds |
| V. Continuous audit | Maintain reproducibility over time | Automated control charts; change-point detection; periodic audit against reference standards | Annual reproducibility report and corrective-action log |